# Appendix A. Complete LLM-assisted SSC elicitation dialogue

## Prompt 1 – Role and EOS-STPA steps

User prompt:

You are an automotive safety engineer with expertise in SOTIF (ISO 21448) and EOS-STPA-based scenario safety analysis. In this conversation, we will analyse an autonomous lane-change scenario using a three-layer EOS-STPA interaction model (ICM, SRM, CSM). We will follow these EOS-STPA-style steps:

- Step E1: Define the analysis framework, losses, and system-level hazards.

- Step E2: Understand the multi-layer interactions between the host vehicle (HV) and external scenario elements (ICM/SRM/CSM).

- Step E3: Identify scenario-specific unsafe control actions (UCAs) for the lane-change manoeuvre.

- Step E4: Derive scenario safety constraints (SSCs) that can prevent or mitigate the UCAs.

When I say “we are at Step E2” or “Step E3”, you should focus on the corresponding task.

Task now: Briefly restate (in 3–4 bullet points) your role and the overall goal of the analysis, using the step names above.

LLM response:

- My role is to act as an automotive safety engineer who applies SOTIF and EOS-STPA to analyse the safety of an autonomous lane-change scenario.

- The overall goal of the analysis is to use the EOS-STPA-style steps (E1–E4) to identify scenario-specific UCAs and derive SSCs that prevent or mitigate system-level hazards and associated losses.

- In Step E1, I will help define and keep consistent the analysis framework, including fixed losses and system-level hazards for the lane-change scenario.

- In Step E2, I will support understanding the multi-layer interactions between the HV and external scenario elements (ICM/SRM/CSM); in Step E3, I will propose lane-change UCAs; and in Step E4, I will help derive corresponding SSCs, always staying within the EOS-STPA framework you provide.

User feedback:

Your understanding of the role and the EOS-STPA steps is correct. Please note the following points for the next steps:

- In Step E1, the losses and system-level hazards are pre-defined by human experts and must be treated as fixed; you should not create new ones.

- In Step E2, focus on interactions between HV and external scenario elements (road level, environment, traffic infrastructure, other vehicles), not on internal controller details.

- In Steps E3 and E4, when you propose UCAs and SSCs, they must always be linked back to the predefined hazards and follow the EOS-STPA-style wording I will show you later.

If this is clear, we can move on to Step E1 and I will provide you with the fixed losses and hazards.

## Prompt 2 – Fixing losses and system-level hazards

User prompt:

We focus on a lane-change scenario. The losses and system-level hazards are defined by human experts as follows and must not be changed.

Losses

- L-1: Personal injury

- L-2: Vehicle losses

- L-3: Mission failed

System-level hazards

- H-1: Collision with target-lane vehicle [L-1][L-2][L-3]

- H-2: Collision with obstacles [L-1][L-2][L-3]

- H-3: Driving out of lane or entering unintended lane [L-3]

- H-4: Rear-ended by vehicle in current/target lane [L-1][L-2][L-3]

- H-5: Collision with facilities [L-1][L-2][L-3]

- H-6: Collision with other vehicles (e.g., motorcycles, etc.) [L-1][L-2][L-3]

Tasks:

- Rewrite each hazard (H-1∼H-6) in one concise English sentence.

- For each hazard, list explicitly which losses (L-1∼L-3) it is linked to.

- Confirm in one sentence that you will not introduce new losses or hazards and will only use L-1∼L-3 and H-1∼H-6 in subsequent steps.

Please output a small markdown table with columns: [Hazard ID, Description, Related Losses].

LLM response:

The hazards and their related losses for the lane-change scenario are summarised below.

| Hazard ID | Description | Related Losses |
| --- | --- | --- |
| H-1 | Collision with a target-lane vehicle during or due to a lane change. | L-1, L-2, L-3 |
| H-2 | Collision with obstacles during lane changing. | L-1, L-2, L-3 |
| H-3 | The HV drives out of lane or enters an unintended lane. | L-3 |
| H-4 | The HV is rear-ended by a vehicle in the current lane or the target lane. | L-1, L-2, L-3 |
| H-5 | Collision with facilities during lane changing. | L-1, L-2, L-3 |
| H-6 | Collision with other vehicles not already covered by H-1 or H-4. | L-1, L-2, L-3 |

I confirm that I will not introduce new losses or hazards and will only use L-1∼L-3 and H-1∼H-6 in subsequent steps.

User feedback:

Your table correctly reflects the predefined hazards and their related losses, and the descriptions are concise and consistent with the lane-change scenario. Please keep the following points in mind for the next steps:

- When you later link unsafe control actions to hazards, pay special attention to H-3, which is only associated with mission failure (L-3), and make sure this is reflected consistently.

- For H-1 and H-4, remember that they are specifically about target-lane vehicles and rear vehicles, which are critical in lane-change scenarios.

If this is clear, we can move on to describing the EOS-STPA three-layer model for the lane-change scenario.

## Prompt 3 – Overview of the three-layer EOS-STPA model

User prompt:

I have already provided you with the EOS-STPA paper. Please read it carefully and understand its underlying mechanism. In EOS-STPA for lane change, we use a three-layer interaction model:

- ICM (Interactive Conceptual Model): high-level closed-loop interactions between the HV and the scenario elements: Road level, Environment, Traffic infrastructure, and Objects.

- SRM (Scenario Refinement Model): semantic decomposition of these elements, e.g., road geometry/topology/boundaries, weather and lighting, traffic signs, etc.

- CSM (Concrete Scenario Model): detailed lane-change control and interaction structure, including the relative motion of target-lane vehicles.

Task: Summarise, in 4–6 bullet points, how these three layers work together for the lane-change scenario.

LLM response:

The three EOS-STPA layers work together in the lane-change scenario as follows:

- The ICM captures the high-level closed-loop interactions between the HV and the external scenario elements: road level, environment, traffic infrastructure, and objects. It shows the basic interaction relationships without going into technical details.

- The SRM refines each scenario element in the ICM into more detailed semantic categories and variables, such as road geometry and lane boundaries, weather and lighting conditions, traffic signs, and the dynamic states of surrounding vehicles.

- The CSM further instantiates the interactions for the lane-change manoeuvre by mapping them onto concrete components and signals, including the lane-change assist system, steering and lighting control, and the relative motion of target-lane vehicles.

- In EOS-STPA analysis, unsafe patterns observed at the CSM level can be linked back to SRM/ICM interactions and finally to system-level hazards and losses, providing a consistent multi-layer view of the scenario.

User feedback:

Your summary of the three EOS-STPA layers is generally correct and consistent with the lane-change scenario. A few clarifications for the following steps:

- When you refer to “objects” at the ICM level, please keep in mind that target-lane front and rear vehicles, as well as rear vehicles in the current lane, are especially important for lane-change safety.

- In the SRM, the refinement of road level (geometry, topology, boundaries) and environment (weather, lighting) is crucial, because they strongly influence perception quality and lane-keeping behaviour.

- In the CSM, we will mainly use the interactions between the Lane Change Assist System and the surrounding vehicles to identify scenario-specific UCAs.

- For the lane-change decision module, in the following steps (especially when identifying UCAs), please pay particular attention to its dependence on the assessment of safe gaps to target-lane front and rear vehicles and rear vehicles in the current lane, as well as on the quality of environment perception.

- The interaction with target-lane and rear vehicles does not issue direct control actions, but it is crucial for scenario safety. In later steps, UCAs related to these vehicles (e.g., changing lanes without ensuring safe gaps, triggering emergency braking during lane change) should be explicitly linked back to this interaction.

- In the subsequent Step E3, when you generate scenario UCAs, you should construct combinations of “control action + scenario context” based on the components, control actions, and feedback signals listed here, rather than inventing new structures that are not grounded in this CSM description.

## Prompt 4 – Initial scenario-specific UCA generation

User prompt:

Now we are at Step E3: Identify scenario-specific UCAs. Based on the fixed losses and system-level hazards (L-1∼L-3, H-1∼H-6) and the CSM-level lane-change control structure we just discussed, please propose candidate scenario-specific UCAs for the autonomous lane-change manoeuvre.

Requirements:

- Focus on the lane-change manoeuvre itself (initiation, execution, and completion).

- Cover unsafe interactions related to perception, decision making, control, and cooperation with other vehicles and infrastructure.

- For each UCA, provide a short English sentence describing the UCA and the hazards H-1∼H-6 that could be triggered.

- To avoid confusion with the expert-identified baseline UCA-1∼UCA-16, please number your candidate LLM-generated UCAs from UCA-17 onward.

Please output UCAs in the format: UCA-xx <description>. [H-x][H-y]…

LLM response:

Here is a set of candidate scenario-specific UCAs for the lane-change manoeuvre:

| UCA ID | Description | Related Hazards |
| --- | --- | --- |
| UCA-17 | HV fails to maintain sufficient perception accuracy for target lane, obstacles, traffic participants and lane markings in any context. | [H-1][H-2][H-3][H-4][H-5][H-6] |
| UCA-18 | HV continues lane change when environment perception is degraded by adverse weather or low visibility. | [H-1][H-2][H-3][H-4][H-5][H-6] |
| UCA-19 | HV does not detect obstacles in the target lane during lane change. | [H-1][H-2][H-6] |
| UCA-20 | Lane-change decision module authorizes lane change when the target lane is unsafe. | [H-1][H-2][H-3][H-4][H-5][H-6] |
| UCA-21 | HV triggers emergency braking during lane change without ensuring rear and lateral safety gaps. | [H-1][H-3][H-4][H-6] |
| UCA-22 | HV initiates lane change when traffic signals or road signs indicate lane change is prohibited or unsafe. | [H-1][H-5][H-6] |
| UCA-23 | HV bases lane-change decision on outdated, delayed or missing traffic infrastructure information. | [H-1][H-3][H-5][H-6] |
| UCA-24 | HV changes lane without maintaining a safe gap to rear vehicles in the current or target lane. | [H-1][H-3][H-4][H-6] |
| UCA-25 | HV performs lane change when lane markings are not reliably detected. | [H-1][H-3][H-6] |
| UCA-26 | HV continues lane change when sudden illumination changes cause unreliable perception of lane and obstacles. | [H-1][H-2][H-3][H-5][H-6] |

User feedback:

Your candidate UCAs cover several important aspects of the lane-change scenario, especially degraded perception, unsafe target-lane conditions, and insufficient gaps. However, for this study I will keep only four of them as new scenario UCAs, because they best complement my existing expert analysis and avoid redundancy:

- UCA-18;

- UCA-20;

- UCA-21;

- UCA-24.

All other candidate UCAs are discarded in this iteration (they are either covered by my baseline UCAs or considered too generic).

Next, I want to renumber and freeze these four UCAs as UCA-17∼UCA-20. Please:

- Renumber the four selected UCAs consecutively as UCA-17, UCA-18, UCA-19, and UCA-20.

- Slightly refine the wording if needed, but keep the original meaning.

Output them in the format: UCA-xx <description>. [H-x][H-y]…

LLM response:

Based on your selection and instructions, the four UCAs are renumbered and slightly refined as follows:

| UCA ID | Description | Related Hazards |
| --- | --- | --- |
| UCA-17 | HV continues lane change when environment perception is degraded by adverse weather or low visibility. | [H-1][H-2][H-3][H-4][H-5][H-6] |
| UCA-18 | Lane-change decision module authorizes lane change when the target lane is unsafe. | [H-1][H-2][H-3][H-4][H-5][H-6] |
| UCA-19 | HV triggers emergency braking during lane change without ensuring rear and lateral safety gaps. | [H-1][H-3][H-4][H-6] |
| UCA-20 | HV changes lane without maintaining a safe gap to rear vehicles in the current or target lane. | [H-1][H-3][H-4][H-6] |

## Prompt 5 – Providing SSC examples and extracting the style pattern

User prompt:

Before generating new SSCs, I will give you some existing EOS-STPA-style constraints as examples.

Existing constraints (subset):

- SSC-11 Sensor feedback shall not provide incorrect distance information when the turn signal is activated. [UCA-11]

- SSC-12 Sensor feedback shall not fail to detect target-lane vehicle information when the turn signal is activated. [UCA-12]

- SSC-13 Sensor feedback shall not provide incorrect target-lane vehicle information when the turn signal is activated. [UCA-13]

- SSC-14 Vehicle action shall not be steering when the turn signal is not activated. [UCA-14]

- SSC-15 Sensor feedback shall not fail to detect target-lane vehicles’ responses when steering is initiated. [UCA-15]

Task: Summarise, in 3–4 bullet points, the style pattern of these constraints (sentence structure, use of “shall/shall not”, how conditions are expressed, and how they link to UCAs). Do not generate new constraints yet.

LLM response:

From SSC-11∼SSC-15, the style pattern of the EOS-STPA scenario safety constraints can be summarised as follows:

- Each constraint starts with a clearly defined subject, such as Sensor feedback or Vehicle action, indicating what is being constrained.

- The constraints use normative language with “shall” or “shall not”, expressing strict requirements rather than recommendations.

- The condition under which the requirement applies is explicitly introduced by “when …”, describing the relevant action or scenario context.

- Each constraint ends with a reference to the corresponding UCA in square brackets (e.g., [UCA-11]), explicitly linking the SSC to the unsafe control action it is intended to prevent or mitigate.

User feedback:

Your summary of the constraint style is correct and matches my EOS-STPA wording. Please keep this pattern when you derive new SSCs:

- clear subject,

- “shall” / “shall not”,

- explicit “when …” condition,

- and a final link to the corresponding UCA in brackets.

Now we can move on to deriving SSC-17∼SSC-20 from UCA-17∼UCA-20.

## Prompt 6 – Deriving SSC-17∼SSC-20 from UCA-17∼UCA-20

User prompt:

Now, using the four new UCAs and the style pattern above, please derive one main SSC for each UCA.

The UCAs are:

- UCA-17 HV continues lane change when environment perception is degraded by adverse weather or low visibility. [H-1][H-2][H-3][H-4][H-5][H-6]

- UCA-18 Lane-change decision module authorizes lane change when the target lane is unsafe. [H-1][H-2][H-3][H-4][H-5][H-6]

- UCA-19 HV triggers emergency braking during lane change without ensuring rear and lateral safety gaps. [H-1][H-3][H-4][H-6]

- UCA-20 HV changes lane without maintaining a safe gap to rear vehicles in the current or target lane. [H-1][H-3][H-4][H-6]

Requirements:

- Generate one constraint per UCA (SSC-17∼SSC-20).

- Use the “subject + shall / shall not + condition with ‘when’ ” structure.

- Each constraint should be able to prevent or mitigate the corresponding UCA.

- End each constraint with the related UCA in brackets, e.g., [UCA-17].

LLM response:

Here are the scenario safety constraints derived from UCA-17∼UCA-20:

| SSC ID | Scenario Safety Constraint | Related UCA |
| --- | --- | --- |
| SSC-17 | HV shall not initiate lane change when environment perception is degraded by adverse weather or low visibility. | [UCA-17] |
| SSC-18 | Lane-change decision module shall not authorize lane change when the target lane is unsafe. | [UCA-18] |
| SSC-19 | HV shall ensure rear and lateral safety gaps before triggering emergency braking during lane change. | [UCA-19] |
| SSC-20 | HV shall not initiate lane change when safe gaps to rear vehicles in the current or target lane are not maintained. | [UCA-20] |

User feedback:

These constraints are acceptable and fully consistent with my EOS-STPA style:

- SSC-17 correctly forbids lane change when perception is degraded by adverse weather or low visibility.

- SSC-18 constrains the lane-change decision module so that it does not authorize lane change when the target lane is unsafe.

- SSC-19 explicitly requires sufficient rear and lateral safety gaps before emergency braking during a lane change.

- SSC-20 ensures that lane change is not executed when safe gaps to rear vehicles in the current or target lane are not maintained.

I will adopt UCA-17∼UCA-20 and SSC-17∼SSC-20 as the final additional scenario UCAs and SSCs generated with LLM support in my EOS-STPA lane-change analysis.